ECA0901 – DIGITAL SIGNAL PROCESSING

C01 – AT3 SIMULATION EXPERIMENT – 6

Design and Analysis of FIR Digital Filters Using Fourier Series and Windowing MATLAB

CODE:

clc;

clear;

close all;

% Input specifications

Fs = 1000;

Fp = 100;

Fst = 150;

Ap = 1;

Ast = 40;

N = 20;

% Cutoff frequency

Fc = (Fp + Fst)/2;

Wn = Fc/(Fs/2);

% Window functions

w1 = ones(N+1,1);

w2 = hamming(N+1);

w3 = hann(N+1);

w4 = blackman(N+1);

% FIR filter design

h1 = fir1(N,Wn,'low',w1);

h2 = fir1(N,Wn,'low',w2);

h3 = fir1(N,Wn,'low',w3);

h4 = fir1(N,Wn,'low',w4);

% Display filter coefficients

disp('Rectangular Window Coefficients');

disp(h1);

disp('Hamming Window Coefficients');

disp(h2);

disp('Hanning Window Coefficients');

disp(h3);

disp('Blackman Window Coefficients');

disp(h4);

% Frequency response

[H1,f] = freqz(h1,1,2048,Fs);

[H2,~] = freqz(h2,1,2048,Fs);

[H3,~] = freqz(h3,1,2048,Fs);

[H4,~] = freqz(h4,1,2048,Fs);

% Magnitude response in dB

figure;

subplot(2,1,1);

plot(f,20*log10(max(abs(H1),1e-6)),'r');

hold on;

plot(f,20*log10(max(abs(H2),1e-6)),'b');

plot(f,20*log10(max(abs(H3),1e-6)),'g');

plot(f,20*log10(max(abs(H4),1e-6)),'k');

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response of FIR Filters');

legend('Rectangular','Hamming','Hanning','Blackman');

xlim([0 Fs/2]);

ylim([-100 5]);

% Phase response

subplot(2,1,2);

plot(f,unwrap(angle(H1)),'r');

hold on;

plot(f,unwrap(angle(H2)),'b');

plot(f,unwrap(angle(H3)),'g');

plot(f,unwrap(angle(H4)),'k');

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (radians)');

title('Phase Response of FIR Filters');

legend('Rectangular','Hamming','Hanning','Blackman');

xlim([0 Fs/2]);

% Individual filter responses

figure;

freqz(h1,1,2048,Fs);

title('Rectangular Window');

figure;

freqz(h2,1,2048,Fs);

title('Hamming Window');

figure;

freqz(h3,1,2048,Fs);

title('Hanning Window');

figure;

freqz(h4,1,2048,Fs);

title('Blackman Window');

% Window comparison

figure;

plot(0:N,w1,'r-o');

hold on;

plot(0:N,w2,'b-o');

plot(0:N,w3,'g-o');

plot(0:N,w4,'k-o');

grid on;

xlabel('Sample index n');

ylabel('Amplitude');

title('Comparison of Window Functions');

legend('Rectangular','Hamming','Hanning','Blackman');

OUTPUT:

OUTPUT PARAMETERS:

RESULT:

Thus, FIR low-pass digital filters were designed using the Fourier-series/windowing method in MATLAB with Rectangular, Hamming, Hanning, and Blackman windows. Their coefficients, magnitude responses, phase responses, and transition characteristics were obtained and compared.