ECA0901 – DIGITAL SIGNAL PROCESSING

C02-A3 SIMULATION EXPERIMENT – 8

Realization of FIR Filters Using Direct, Cascade and Linear-Phase Structures Using MATLAB

CODE:

clc; clear; close all;

% FIR coefficients (symmetric)

h = [1 2 3 4 3 2 1]/16;

N = length(h)-1;

Fs = 1000;

x = [1 zeros(1,127)];  % Unit impulse

% Direct realization

y1 = filter(h,1,x);

% Cascade realization using polynomial factorization

r = roots(h);

p = poly(r);

y2 = filter(p,1,x);

% Symmetry and linear-phase check

disp('FIR Coefficients:'); disp(h);

disp('Coefficients are symmetric:');

disp(isequal(h,fliplr(h)));

disp('Maximum output difference:');

disp(max(abs(y1-y2)));

% Impulse and output responses

figure;

subplot(2,1,1);

stem(0:127,y1,'filled'); grid on;

title('Direct and Cascade Impulse Responses');

xlabel('n'); ylabel('Amplitude');

hold on;

stem(0:127,y2,'r');

% Phase response

[H,f] = freqz(h,1,1024,Fs);

subplot(2,1,2);

plot(f,unwrap(angle(H))); grid on;

title('Linear Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (radians)');

% Output comparison

figure;

plot(0:127,y1,'b-o'); hold on;

plot(0:127,y2,'r--');

grid on; legend('Direct','Cascade');

title('Output Comparison');

xlabel('n'); ylabel('Output');

OUTPUT:

OUTPUT PARAMETERS:

RESULT:

The FIR filter was realized using direct and cascade structures. Symmetric coefficients and linear-phase behaviour were verified, and both outputs were compared to confirm functional equivalence..