ECA0901 – DIGITAL SIGNAL PROCESSING

C01 – AT3 SIMULATION EXPERIMENT – 4

Design and Comparative Analysis of Butterworth, Chebyshev and Elliptic IIR Filters Using MATLAB

MATLAB CODE:

clc; clear; close all;

% Input specifications

Fs = 1000; Fp = 100; Fst = 150; Rp = 1;

Rs = 40; Wp = Fp/(Fs/2);

Ws = Fst/(Fs/2); BW = Fst - Fp;

% Minimum filter orders

[Nb,Wb] = buttord(Wp,Ws,Rp,Rs);

[Nc,Wc] = cheb1ord(Wp,Ws,Rp,Rs);

[Ne,We] = ellipord(Wp,Ws,Rp,Rs);

% Design filters

[bb,ab] = butter(Nb,Wb,'low');

[bc,ac] = cheby1(Nc,Rp,Wc,'low');

[be,ae] = ellip(Ne,Rp,Rs,We,'low');

% Display orders and coefficients

fprintf('Butterworth Order = %d\n',Nb);

disp('Butterworth Numerator:'); disp(bb);

disp('Butterworth Denominator:'); disp(ab);

fprintf('Chebyshev-I Order = %d\n',Nc);

disp('Chebyshev Numerator:'); disp(bc);

disp('Chebyshev Denominator:'); disp(ac);

fprintf('Elliptic Order = %d\n',Ne);

disp('Elliptic Numerator:'); disp(be);

disp('Elliptic Denominator:'); disp(ae);

% Frequency responses

[Hb,f] = freqz(bb,ab,2048,Fs);

[Hc,~] = freqz(bc,ac,2048,Fs);

[He,~] = freqz(be,ae,2048,Fs);

dB = 20*log10(abs(Hb)+eps);

dC = 20*log10(abs(Hc)+eps);

dE = 20*log10(abs(He)+eps);

%Magnitude response comparison

figure;

plot(f,dB,'b',f,dC,'r--',f,dE,'k-.');

grid on; ylim([-100 5]);

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response Comparison');

legend('Butterworth','Chebyshev-I','Elliptic');

% Phase response comparison

figure;

plot(f,unwrap(angle(Hb)),'b', ...

f,unwrap(angle(Hc)),'r--', ...

f,unwrap(angle(He)),'k-.');

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (rad)');

title('Phase Response Comparison');

legend('Butterworth','Chebyshev-I','Elliptic');

% Measure passband ripple and stopband attenuation

pass = f <= Fp;

stop = f >= Fst;

rippleB = max(dB(pass))-min(dB(pass));

rippleC = max(dC(pass))-min(dC(pass));

rippleE = max(dE(pass))-min(dE(pass));

attB = -max(dB(stop)); attC = -max(dC(stop)); attE = -max(dE(stop));

% Percentage deviation from specifications

devB = abs(attB-Rs)/Rs*100;

devC = abs(attC-Rs)/Rs*100;

devE = abs(attE-Rs)/Rs*100;

% Comparative table

Filter = {'Butterworth';'Chebyshev-I';'Elliptic'};

Order = [Nb;Nc;Ne];

PassbandRipple_dB = [rippleB;rippleC;rippleE];

StopbandAttenuation_dB = [attB;attC;attE];

StopbandDeviation_percent = [devB;devC;devE];

Results = table(Filter,Order,PassbandRipple_dB,...

StopbandAttenuation_dB,StopbandDeviation_percent);

disp(Results);

fprintf('Transition Bandwidth = %.2f Hz\n',BW);

OUTPUT:

OUTPUT PARAMETERS:

RESULT:

Design and Comparative Analysis of Butterworth, Chebyshev and Elliptic IIR Filters satisfied outcomes using MATLAB