ECA0901 – DIGITAL SIGNAL PROCESSING

C01 – AT3 SIMULATION EXPERIMENT – 4

Design and Comparative Analysis of Butterworth, Chebyshev and Elliptic IIR Filters Using MATLAB

MATLAB CODE:

clc; clear; close all;

% Design stable IIR filter

[b,a] = butter(4,0.3,'low');

x = [1 zeros(1,49)];

% 1. Direct Form-I

y1 = filter(b,a,x);

% 2. Direct Form-II

y2 = filter(b,a,x);

% 3. Cascade realization

[sos,g] = tf2sos(b,a);

y3 = g*x;

for i = 1:size(sos,1)

y3 = filter(sos(i,1:3),sos(i,4:6),y3);

end

% 4. Parallel realization

[r,p,k] = residuez(b,a);

[br,ar] = residuez(r,p,k);

y4 = filter(br,ar,x);

% Display coefficients

disp('Numerator:'); disp(b);

disp('Denominator:'); disp(a);

disp('Second-Order Sections:'); disp(sos);

disp('Parallel Residues:'); disp(r);

disp('Parallel Poles:'); disp(p);

% 5. Compare responses

figure;

plot(0:49,y1,'b',0:49,y2,'r--',...

0:49,y3,'g:',0:49,y4,'k-.');

grid on;

xlabel('Samples'); ylabel('Amplitude');

title('IIR Realization Comparison');

legend('Direct Form-I','Direct Form-II','Cascade','Parallel');

fprintf('DF-I vs DF-II error = %.2e\n',max(abs(y1-y2)));

fprintf('DF-I vs Cascade error = %.2e\n',max(abs(y1-y3)));

fprintf('DF-I vs Parallel error = %.2e\n',max(abs(y1-y4)));OUTPUT:

OUTPUT PARAMETERS:

RESULT:

Thus, the IIR filter was realized using Direct Form-I, Direct Form-II, Cascade, and Parallel structures in MATLAB, and their output responses were verified.